Arkansas ARES/RACES - Packet message handling

WHEN Packet... "Anytime you want to shove the paper through the radio..."

- Lengthy, detailed, non-immediate, multiple relay

* Lots of numbers

* Numerous words requiring spelling

* Multiple relays to destination

* Lists of items or names

Voice... "Anytime the message has to be there now, and can be stated simply..."

- Brief, urgent, concern over delivery, content easily understood

* Notification of disasterous event: tornado, funnel cloud, explosion

* Immediate request for aid or assistance

* Threat to life or property

* direct communication or single relay

HOW Packet...

- Method:	Type into file, if possible, then transmit to BBS or Mailbox

- Addressing:	SP W5AUU   subject: Disaster relief request - Baxter County

- Style:		either formal or informal; all messages to ADEM need signature of official

- Reply:	Include return routing, either BBS or Mailbox

*BBS:  K5BAX @ KD5AYE.#NEAR.AR

*Mailbox: K5BAX <= BAXTER node

- Delivery:	Deliver messages timely; do not send expired messages

- Records:	Keep copies of ALL messages!!

- Operator:	Appropriately trained personell

* Should be familiar with YOUR particular terminal program setup

* Should become familiar with Network connections prior to exercise

* Should have some typing and computer skills

* Operator may be able to perform another selected task

- Network:	Configure for best usage of  network

* Connect to nearest node, allow it to autoroute to distant node

* NODE command tells you which distant nodes it can route to

* Do not connect to backbone directly using a 6 meter radio

* Do not use digipeating or "Via"; connect to node.

* Simplex communication on Node frequency still ties up the node

* Be patient with network; traffic can temporarily slow or halt it

WHY Packet...

- Packet takes overflow from voice network (if no messages, feel lucky!)

* Keeps from overworking fatigued voice operators

* Keeps channel clear for urgent messages

- Allows more numerous and more detailed information, not previously provided

- Capability of both local and out of area message delivery or relay

Packet is not a do all, be all... use it as just another radio resource, no more, no less...